ECO* PLUSE  |  Typical Specifications

File No: 90.203

Date: July 24, 2013

Supersedes: NEW

Date: NEW

General Description

HVAC Health Management Service

- Commissioning - diagnostic services

      - The chilled water plant automation solution shall be provided with ___ year (1 year minimum) of continuous diagnostic and health management service covering the central chilled water plant.

      - The diagnostic and health management service solution will be offered as software package as a part of the optimization solution.

      - The diagnostic service shall provide building owners and operators insight on recommendations to keep their cooling system running at an optimal level throughout the year(s).

      - The diagnostic service shall provide daily emails that summarize performance and identify possible issues by tracking the operation of the equipment and of the overall system.

      - The diagnostic service shall provide recommended remedies accessible through an intuitive interface view via web interface.

      - Separate from the online reporting, the diagnostic services shall include email summaries of regular reviews and quarterly reports directly from the manufacturer’s expert reporting on all the key findings during the operating period.

      - The report shall provide analysis based on the following metrics:

- Chiller system overall efficiency

- Equipment specific performance

- Financial savings

- Environmental impact

- Indicators of comfort

      - Automatic email notification will include measured actual performance relative to the base case and the predicted case.

      - The automatic email summary of the notifications shall be customizable to fit the project specific’s need of each central cooling system. The following are the suite of expected notification.

- Cooling tower low flow

- Cooling tower high water consumption

- Cooling tower max sump level exceeded

- Cooling tower below minimum sump level

- Cooling tower high leaving water temperature

- Chiller high evaporator approach

- Chiller high condenser approach

- Chiller excessive run hours

- Unstable chilled water supply

- Low delta t syndrome

- Chiller efficiency below expectations

- Pump rpm different than recommended

- Cooling tower trip on vibration

- Compressor excessive vibration

- Pumps excessive vibration

- High pump kW relative to chiller kW